ORGAN FIXATION

Fixation of Tissues for Routine Paraffin Histology

- Trim tissues to contain needed anatomy and to minimal thickness prior to immersion fixation.

• Trim tissues with a razor blade using a single slicing motion; avoid crushing or sawing motion.

• Trim tissues to make plane of section obvious and well exposed to fixative.

• Trim tissues to not exceed 2.5mm thickness

• Trim tissues to remove any barrier membranes, which might slow fixation (ie. cortex of kidney, skin from muscle, etc.)

• Perforate capsular membranes with a fine injection needle (ie. eye and testis).

- Place tissues quickly into fixative following trimming. Adjust fixative volume to be at least 20:1 excess of the estimated tissue volume.

- Fix tissues at room temperature for 2 days with continuous agitation.

- Transfer tissues to a static buffer or dehydrating solution following fixation, such as PBS or 50% ethanol.

- Submit samples for paraffin processing without delay.

Fixation of Tissues for Frozen Histology

- Trim tissues to contain needed anatomy and to minimal thickness prior to immersion fixation.

• Trim tissues with a razor blade using a single slicing motion; avoid crushing or sawing motion.

• Trim tissues to make plane of section obvious and well exposed to fixative.

• Trim tissues to not exceed 2.5mm thickness

• Trim tissues to remove any barrier membranes, which might slow fixation (ie. cortex of kidney, skin from muscle, etc.)

• Perforate capsular membranes with a fine injection needle (ie. eye and testis).

- Place tissues quickly into fixative following trimming. Adjust fixative volume to be at least 20:1 excess of the estimated tissue volume.

- Fix tissues at room temperature for 2 days with continuous agitation.

- Transfer tissues to 10% sucrose/1xPBS and equilibrate for 12 hours at 4°C.

- Replace 10% sucrose/1xPBS with 18% sucrose/1xPBS and equilibrate for an additional 12 hours at 4°C.

- BRAIN ONLY; further exchange 18% sucrose/1xPBS with 30% sucrose/1xPBS and equilibrate for additional 24 hours at 4°C.

- Submit samples for cryoembedding without delay.

Fixation of Tissues for Immunohistochemistry and In-Situ Hybridization

- Collect tissues with care to avoid RNAse and proteinase contamination following whole animal transcardial perfusion with ice-cold heparinized saline and subsequent fixative of choice (ice-cold 4% paraformaldehyde/ DEPC-PBS, ph7.4 in most cases).

- Trim tissues to contain needed anatomy and to minimal thickness prior to continued overnight fixation by immersion.

• Trim tissues with a razor blade using a single slicing motion; avoid crushing or sawing motion.

• Trim tissues to make plane of section obvious and well exposed to fixative.

• Trim tissues to not exceed 2.5mm thickness

• Trim tissues to remove any barrier membranes, which might slow fixation (ie. cortex of kidney, skin from muscle, etc.)

• Perforate capsular membranes with a fine injection needle (ie. eye and testis).

- Place tissues quickly into fixative following trimming. Adjust fixative volume to be at least 20:1 excess of the estimated tissue volume.

- Fix tissues at 4°C for 2 –16 hours with continuous agitation depending upon size, integrity and lipid content.

- Transfer tissues to a static buffer or dehydrating solution following fixation, such as DEPC-PBS or 50% ethanol.

- Submit samples for paraffin processing without delay.